# Group 16 Design Document: Battery Temperature Monitor

Oct 22, 2026 · @Irei Hashimoto

## 1. Overview

This system monitors battery-cell temperatures, reports the hottest reading over I2C, and flags a fault above 60°C. Each temperature sensor produces a voltage that falls as temperature rises. A multiplexer connects one sensor at a time to a signal-conditioning stage, and the Arduino Nano's 10-bit ADC converts that voltage to a number. The firmware converts each reading to a temperature, finds the maximum across all channels, and sends it over I2C. A blinking red LED shows the system is running and indicates a fault. A 12 V to 5 V DC-DC converter powers the board from the car's low-voltage battery.

Figure 1 Block Diagram

## 2. Requirements and assumptions

The project requires a circuit that reads 16 temperature sensors, converts them to digital values, and reports the highest temperature over I2C.

Requirements

- 16 temperature sensors

- A CD74HC4067 analog multiplexer (schematic); CD4051BE in physical build

- A signal conditioning stage between the multiplexer and the Nano

- An Arduino Nano that converts each reading with its ADC and finds the highest temperature

- I2C output of that temperature

- A blinking red LED, with a fault indicated above 60°C

- A 12 V to 5 V DC-DC converter (schematic)

Assumptions

- Sensor supply: 5V through 680 Ω, per the datasheet test circuit.

- The LV battery is nominally 12 V.

- The physical build reads 8 sensors through the CD4051BE, per the leads' update.

- ADC reference: default 5 V; conversion formula uses 1024.

## 3. Hardware

### 3.1 Power

| Load | Current (mA) | Source |
| --- | --- | --- |
| Sensors (worst case, 120°C) | 16mA |  |
| Arduino Nano | 50mA |  |
| Mux | 8 µA |  |
| LED | 232 mA |  |
| I2C pull-ups | 3.3 mA |  |
| Total |  |  |

### 3.2 Temperature sensors

- Each sensor is in series with a 680 Ω resistor between the 5 V rail and ground, which matches the test circuit in the sensor datasheet [1]. We match it because the datasheet's voltage-to-temperature table is only valid under those conditions.

- The resistor and sensor form a voltage divider, and the measured signal is the voltage at the point between them. As the sensor heats, its effective resistance falls, so its share of the 5 V falls. The sensor's voltage therefore drops as temperature rises.

Table 1: Sensor output at key temperatures (from datasheet Table 5 [1])

| Temperature (°C) | Sensor voltage (V) | ADC reading (5 V ref) |
| --- | --- | --- |
| -40 | 2.44 | 499 |
| 0 | 2.17 | 444 |
| 25 | 1.86 | 380 |
| 60 | 1.51 | 309 |
| 120 | 1.3 | 266 |

### 3.3 Multiplexer

- MUX selects a channel from the binary code sent from the Arduino. It follows this truth table:

- The mux has a ON resistance of about 160 ohms at worst and 70 ohm typical

- Settling delay can be calculated with the equation:

Settling delay = N(R(source) + R(on)) x C(total) = ~1 ns

N = 6.93 time constants (τ)

Rsource = 670 Ohms

Ron = 160 Ohms

C 14 pF nano

- To solve the issue of reading this settling delay a software delay of 1 nanosecond is added.

### 3.4 Signal conditioning

- The sensors would produce a voltage from 0-~2.5V, however our Arduino nano is set for a 5v range. This creates a signal that isn't as accurate from the lower resolution.

- To solve this there is an non-inverting Operational Amplifier that amplifies voltage to a gain of 2.

### 3.5 Nano and ADC

- ADC: 10-bit, 5 V reference, LSB 4.88 mV.

### 3.6 Fault LED

### 3.7 I2C

- The I2C circuit is pulled up to 1.5k ohms. This is because the lower the resistance the less RLC noise would be caused from the added impedance to the circuit.

## 4. Software

The firmware (software/temp_monitor/temp_monitor.ino) runs on the Arduino Nano. Each pass of loop() reads every channel, converts each reading to a temperature, finds the maximum, checks it against the 60 °C threshold, updates the LED, and sends the maximum over I2C.

Figure 2: Firmware flow chart

### 4.1 Program structure

setup() runs once to configure the serial port, the output pins (mux select lines, mux enable and LED) and I2C. loop() then repeats indefinitely:

- For each channel i from 0 to NUM_CHANNELS − 1: select channel i on the mux, read the analog pin with analogRead(), convert the reading to a temperature, and store it in slot i of the temperature array (1D array).

- Find the highest temperature in the array.

- Determine whether it exceeds 60 °C.

- Update the LED (normal blink, or fault pattern).

- Send the highest temperature over I2C.

Functions:

- channelSelector(channel): sets the MUX select lines to the binary value of the channel. Returns nothing.

- readingToTemp(reading): converts an ADC reading to °C by calling stepsToVoltage, then voltageToTemp.

- findHighest(temperatures, count): returns the highest temperature in the array.

- checkLed(temperature): returns true if the temperature in the argument is above 60 °C.

- ledControl(fault): controls the LED. Returns nothing.

- i2cSend(temperature): sends the highest temperature over I2C. Returns nothing.

- analogRead(pin): built-in Arduino function [3]. Reads the voltage on the given analog pin (the mux output, MUX_PIN) with the 10-bit ADC and returns a value from 0 to 1023.

Helper functions:

- stepsToVoltage(reading): converts an ADC reading to the sensor voltage: V = reading × V_ref ÷ 1024 ÷ 2, where 2 is the gain of the signal conditioning stage.

- voltageToTemp(voltage): converts volts to °C using the datasheet table.

### 4.2 Design decisions

- One value per conversion function. Each conversion helper takes one value and returns one value, so each can be tested against hand calculations: for example, a reading of 800 should give 1.9531 V and 17.63 °C. loop() stores each result in the array.

- Sensor table as a constant. The 33 datasheet voltages (−40 °C to 120 °C in 5 °C steps) are stored once as a const array. Temperatures are evenly spaced, so a table index converts directly to a temperature and only voltages need storing [1].

- Channel count as a single constant [4], [5]. NUM_CHANNELS sizes the array and bounds the loop. It is 16 for the CD74HC4067 schematic and 8 for the CD4051BE build, so switching requires one edit. The CD4051BE also uses 3 select lines instead of 4, so the select-line pin constants change with it.

- Temperature array passed with its count. findHighest receives the array and the count as arguments rather than reading a global, so it works for any channel count and can be tested with made-up arrays.

- The sensor is non-linear, and the datasheet recommends linear interpolation between table values for reasonable accuracy, so we used it [1].

- We considered a 2D array of temperature–voltage pairs, but because the temperatures are evenly spaced (T = −40 + 5k), they are computed from the table index, so only the voltages need storing.

- ADC conversion uses 1024 [2], matching the ATmega328P datasheet's conversion formula. The difference from 1023 is about 0.1%.

- MUX settling. After switching channels, the firmware waits briefly before reading so the output reflects the newly selected sensor (Section 3.3).

### 4.3 Math explanation

Step 1: ADC reading to sensor voltage. The Nano's ADC is 10-bit, so a reading is numbered from 0 to 1023, with a 5 V reference [2]. One count (LSB) is 5 ÷ 1024 = 4.88 mV. The signal conditioning stage multiplies the sensor voltage by 2, so:

V_ADC = reading × 5 ÷ 1024

V_sensor = V_ADC ÷ 2

Step 2: Voltage to temperature (linear interpolation). The sensor table lists voltages from −40 °C to 120 °C in 5 °C steps. Table index k corresponds to:

T(k) = −40 + 5k

The firmware finds the two entries the voltage falls between, V(k) ≥ V > V(k+1), and interpolates linearly:

fraction = (V(k) − V) ÷ (V(k) − V(k+1))

T = T(k) + 5 × fraction

Because the sensor's voltage falls as temperature rises, V(k) is the higher voltage and T(k) the lower temperature.

Worked example: reading 800

- V_ADC = 800 × 5 ÷ 1024 = 3.9063 V

- V_sensor = 3.9063 ÷ 2 = 1.9531 V

- Bracket: 15 °C (1.99 V) and 20 °C (1.92 V)

- fraction = (1.99 − 1.9531) ÷ (1.99 − 1.92) = 0.0369 ÷ 0.07 = 0.527

- T = 15 + 5 × 0.527 = 17.63 °C (no fault)

### 4.4 Still to do for the build phase

- Implement and test against calculations.

- Handle readings outside the table range (sensor voltage below 1.30 V or above 2.44 V). These indicate a disconnected or shorted sensor, and interpolation has no bracket outside the table, so they must be detected and reported.

## 5. Testing and limitations

Testing results will be added after the build phase.

## 6. References

AI tools were used for planning, research, learning, drafting, and support; all code was written by the authors.

- enepaq, Li-ion building block with Sony/Murata VTC5A, datasheet, rev. E, Jan. 2025. Table 5: sensor voltage vs. temperature.

- Microchip Technology, ATmega328P, datasheet. Analog-to-Digital Converter chapter: conversion result formula.

- Arduino, Language Reference: analogRead(), digitalWrite(), pinMode(), bitRead(), millis().

- Texas Instruments, CD74HC4067, CD74HCT4067 16-Channel Analog Multiplexer/Demultiplexer, datasheet SCHS209D, Dec. 2024.

- Texas Instruments, CD4051B, CD4052B, CD4053B CMOS Analog Multiplexers/Demultiplexers, datasheet SCHS047L, Sep. 2023.